IMPLEMENTATION OF SOLAR-POWERED WATER PUMP AND MONITORING SYSTEM FOR HYDROPONIC URBAN FARMING

Adi Mahmud Jaya Marindra1*, Dwi Surya Santaki2, Deny Kurniawan3, Ferdian Ilham Maulana4, Raihan Adji Widjaksana5, Muhammad Irham Amzari6, Pandu Jati Bayu Saputra7, Chenny Azalia Eka Putri Minggus8, Rafi Fauzan9

1,2,3,4,5 Program Studi Teknik Elektro, Institut Teknologi Kalimantan

6,7 Program Studi Teknik Mesin, Institut Teknologi Kalimantan

8,9 Program Studi Teknik Industri, Institut Teknologi Kalimantan

email: adi.marindra@lecturer.itk.ac.id

Abstract: Hydroponic farming is gaining popularity in urban areas due to its efficient resource usage and high, clean yields. However, systems like the Nutrient Film Technique (NFT) rely on electric pumps for continuous nutrient circulation, making them vulnerable to extended power outages that can disrupt plant nutrition and lead to crop failure. This community service project addresses that issue by implementing a solar-powered water pump and a water quality monitoring system in the hydroponic gardens of the Women Farmers Group (KWT) in Sepinggan Raya, Balikpapan. The program consisted of an initial survey, system development and installation, and training sessions on system operation and maintenance. Results indicate that the implementation of the solar pump and monitoring system improves hydroponic productivity while reducing operational costs previously dependent on the national electricity grid. Additionally, the project empowered the local women's group with renewable energy knowledge and modern farming technologies, promoting sustainable urban agriculture practices.

Keywords: Hydroponic; renewable energy; application of technologies; urban farming; water quality monitoring

Abstrak: Pertanian hidroponik semakin populer di area perkotaan yang memiliki keterbatasan ruang karena efisiensi sumber daya dan hasil panen yang tinggi. Namun, sistem hidroponik seperti Nutrient Film Technique (NFT) sangat bergantung pada pompa listrik untuk sirkulasi air nutrisi secara berkelanjutan. Pemadaman listrik yang berkepanjangan dapat mengganggu suplai nutrisi dan berpotensi menyebabkan kegagalan panen. Proyek pengabdian kepada masyarakat ini bertujuan mengatasi kerentanan tersebut dengan mengimplementasikan pompa air tenaga surya dan sistem pemantauan kualitas air berbasis sensor di kebun hidroponik Kelompok Wanita Tani (KWT) Seraya, Sepinggan Raya, Balikpapan. Kegiatan meliputi survei awal, pengembangan dan instalasi sistem, serta pelatihan pengoperasian dan pemeliharaan teknologi bagi para petani. Hasil menunjukkan bahwa sistem ini meningkatkan produktivitas kebun hidroponik dan mengurangi ketergantungan terhadap jaringan listrik nasional (PLN). Selain itu, proyek ini memberdayakan kelompok tani dengan pengetahuan tentang energi terbarukan dan teknologi pertanian modern, mendorong praktik pertanian yang berkelanjutan di lingkungan urban.

Kata kunci: Energi baru terbarukan; hidroponik; monitoring kualitas air; penerapan teknologi; urban farming

INTRODUCTION

Hydroponic farming has been known since the 17th century when Francis Bacon first documented the concept in 1627 (Caputo, 2022). In Indonesia, hydroponics was popularized in the 1980s through Bob Sadino’s initiatives (Setyo et al., 2023). Originally adopted as a hobby by plant enthusiasts eager to experiment with soil-less cultivation, hydroponics has since evolved into a significant commercial agricultural practice, especially in urban areas where limited space restricts traditional farming (Monisha et al., 2023; Serio et al., 2022). This transformation demonstrates hydroponics’ adaptability and efficiency in densely populated environments (Debangshi, 2021). Furthermore, the COVID-19 pandemic accelerated hydroponic entrepreneurship, emphasizing sustainable income with economic, social, and environmental considerations (Pardede & Pardede, 2022).

In Balikpapan City, the KWT Seraya RT 28 exemplifies urban farming’s potential in East Kalimantan (Anggraini, 2022). Their farming system integrates hydroponic methods, especially the Nutrient Film Technique (NFT), and traditional soil-based cultivation. Lettuce is the primary hydroponic crop due to its market demand and economic value (Agrawal et al., 2020). Additional crops include spinach, celery, bok choy, chili, eggplant, and fruit trees such as Sunkist oranges and Crystal guava (Putri & Matarru, 2023). Despite being in its early stages, the hydroponic initiative has carved a niche market.

Maintaining irrigation for hydroponic plants is crucial to prevent root drying and wilting. For NFT-type hydroponics, the continual operation of the nutrient pumps during the night is essential to sustain these moisture levels and provide necessary nutrients (Maestre-Valero et al., 2018). Unfortunately, in Sepinggan Raya, the farming community faces significant challenges due to power outages. Frequent power outages interrupt the electric pumps essential for nutrient delivery, risking plant wilting and loss. Furthermore, water quality checks, such as pH measurement using well water, are manually performed, consuming time and reducing precision. This situation highlights the interplay between the physical needs of hydroponic farming and the socio-economic environment (Ragaveena et al., 2021).

Recent advances in renewable energy, electronics, and sensor technologies offer solutions to these issues (Majeed et al., 2023; Ng & Mahkeswaran, 2021). Solar energy addresses power disruptions by ensuring consistent pump operation (Gorjian et al., 2021), while automated sensors can optimize nutrient levels, pH, and moisture, reducing the risks of manual monitoring (Prakash et al., 2023). Without a stable power supply, these systems can automate and optimize nutrient delivery, pH balance, and moisture levels (Saad et al., 2021). The ability to closely monitor and adjust these parameters can mitigate the risks associated with manual checks and the uncertainties of power-reliant systems.

Community services play a crucial role in introducing technologies to the society. In previous studies, training in hydroponics improves yields and empowers local farmers with modern skills (Rozak et al., 2022). Additionally, installing solar-powered public lighting in community areas enhances safety and accessibility while promoting environmental sustainability (Marindra et al., 2022). Furthermore, plant monitoring technology, including sensors and data analytics tools, allows for precise control over environmental conditions, leading to optimized plant growth and reduced waste (Nasution et al., 2021).

This project aims to implement two innovations in the KWT Seraya community. First, we introduce a solar-powered water pump system to maintain consistent irrigation during power outages. This ensures that the hydroponically grown lettuce and other crops do not suffer from interrupted water supply. Second, we deploy a water quality monitoring system focusing on pH level management, which automates the monitoring process, reduces manual intervention, and maintains optimal nutrient conditions for plant growth. These initiatives address immediate challenges in Sepinggan Raya and contribute to the broader goal of sustainable urban farming.

METHOD

Figure 1 illustrates the method of our community service project. The community service project was conducted through five main stages. The first stage was an initial survey to identify key problems, particularly power outages that affected irrigation continuity and the manual water quality monitoring process. Based on the findings, the second stage involved the development of a solar-powered water pump and a water quality monitoring system. In the third stage, both systems were installed on-site to provide a reliable irrigation source and automate pH and nutrient level monitoring. The fourth stage focused on training and capacity building for the Women Farmers Group (KWT Seraya), ensuring they could operate and maintain the new technologies independently. Finally, an evaluation and feedback phase were implemented, involving direct observation and farmer input to assess the effectiveness of the systems and make necessary adjustments.

Figure 1. Method of community service

Figure 2. Location of community service at Seraya Beach Balikpapan seen from ITK

The community service project was located at the hydroponic facility in Pantai Seraya, Sepinggan, in the district of South Balikpapan, Balikpapan City, East Kalimantan. As depicted in Figure 2, the location is close to the ITK campus, approximately 24.1 kilometers away, roughly a 31-minute drive via the Balikpapan-Samarinda toll road. This proximity facilitated ease of access for the project team and stakeholders, allowing for efficient coordination and implementation of the community service activities.

Figure 3. Diagram of the implemented solar-powered pump and pH/nutrient monitoring system

Figure 3 presents a diagram of the solar-powered water pump and pH-nutrient monitoring systems implemented at the hydroponic farm. The pump system harnesses solar energy via photovoltaic panels, with energy stored in batteries and regulated by a solar charge controller. An inverter converts direct current (DC) to alternating current (AC) to operate the water pump, while an Automatic Transfer Switch (ATS) enables a seamless transition between solar and grid electricity, ensuring uninterrupted irrigation. The pH and nutrient monitoring system, essential for maintaining optimal plant nutrition, is centered on an Arduino UNO microcontroller that receives continuous data from pH and nutrient probes. This data is displayed in real-time, enabling precise adjustments. A submersible pump circulates the nutrient solution using solar power, demonstrating the farm’s commitment to sustainability and technological advancement. This integrated system addresses challenges in power reliability and water quality control, setting a model for autonomous, renewable energy-based hydroponic farming.

The success and impact of the community service project at RT 28, Sepinggan Raya, which focused on implementing a solar-powered water pump and a hydroponic nutrient and pH monitoring system, were evaluated through a structured survey. The survey collected feedback from the farmers on several key aspects: the usefulness of the developed systems; the effectiveness of the solar-powered pump during PLN outages; the ease of hydroponic water management using the monitoring system; the contribution of the technologies to enhancing the appeal of the hydroponic site at Seraya Beach; the perceived improvement in vegetable quality and production; and the performance of the community service team in executing planned activities and achieving project objectives.

DISCUSSION

The project began with a field study (Figure 4) to understand local conditions and farming practices. Surveys and observations of hydroponic and soil-based systems (Figure 4a) were conducted, along with discussions involving the neighborhood unit leader and local farmers (Figure 4b). This initial assessment identified key problems such as irrigation failure due to outages and the inefficiency of manual water quality checks.

The realization of these innovations began with determining the type of pump being used, which then informed the specifications for the photovoltaic (PV) modules to be used. Following this, the procurement and assembly of the necessary equipment were undertaken. The assembly included components like the Automatic Transfer Switch (ATS), a 1000-Watt PSW Inverter, a Solar Charge Controller (SCC), terminal blocks, MCB rails, indicator lights, and the hydroponic monitoring system including an LCD and various supporting components. Installation involved placing 170 Wp solar panels and relevant sensors at the farm.

(a)

(b)

Figure 4. Initial survey: (a) The hydroponic and soil-based farms (b) Discussions with the head of the neighborhood unit and a local farmer

Figure 5a illustrates the assembly and development process, showing key equipment testing and integration. Upon completion, the system was installed at the farm. The solar panel was mounted on the rooftop beside the hydroponic area, while the control and monitoring panel was installed at the farm. A two-week testing period confirmed that the PLTS could power the water pump and nutrient-pH monitoring system for over two hours beyond expected durations, indicating the success of the implemented technologies.  Figure 5b illustrates the training and capacity-building phase, essential to ensure sustainability and effective system use. A hands-on session was conducted for the women farmers to operate the solar-powered pump and the monitoring system independently. This training equipped them with practical skills and strengthened their ability to adopt technologies in hydroponics farming.

(a)

(b)

Figure 5. System installation and training: (a) Developing the system onsite (b) Training activity

During the training sessions, participants were thoroughly guided through the standard operating procedures of the solar-powered system. They were instructed on monitoring battery voltage via the Solar Charge Controller (SCC), emphasizing maintaining a minimum of 10 volts for system operation. Procedures for activating the inverter, handling the Miniature Circuit Breaker (MCB), and manually switching from battery to grid power were explained, including the SCC's automatic disconnection and reset functions.

Figure 6. Evaluation results of the community service project

A survey involving 22 hydroponic farmers was conducted to evaluate the project's impact, with results presented in Figure 6. The average score was 3.79 out of 4, indicating that implementing the solar-powered pump and the monitoring system was highly successful and beneficial. Respondents reported improvements in irrigation reliability during PLN outages, ease of water quality management, and overall system efficiency. The survey also highlighted the increased attractiveness of the hydroponic site, better crop quality and yield. These outcomes validate the project's effectiveness in addressing local challenges and empowering KWT Seraya with sustainable, technology-driven agricultural solutions for future urban farming.

CONCLUSION

Implementing a solar-powered water pump and monitoring system in Sepinggan Raya’s hydroponic urban farming has significantly improved agricultural productivity and sustainability. The solar pump addressed irrigation disruptions caused by electricity outages, while the monitoring system enabled automated control of pH and nutrient levels, maintaining optimal growing conditions. The high survey score reflects the project's success, indicating strong community approval. Training and capacity-building activities strengthened KWT Seraya’s ability to adopt and maintain the technology. Overall, this initiative resolved immediate challenges and established a foundation for future advancements in sustainable urban farming.

ACKNOWLEDGMENT

The authors thank the Institute of Research and Community Service (LPPM) of Institut Teknologi Kalimantan (ITK) for financially supporting this community service project. We also thank Mrs. Tatu Neni Suryani and the Women Farmers Group of RT. 28 Sepinggan Raya, Balikpapan, for supporting the community service team.

BIBLIOGRAPHY

Agrawal, R., Gandhi Krishi Vishwavidyalaya, I., Tripathi, M., Verma, A., & Khalkho, D. (2020). Hydroponic systems for cultivation of horticultural crops: A review. Journal of Pharmacognosy and Phytochemistry, 9(6), 2083–2086.

Anggraini, M. A. (2022, February 17). Bantu Kelompok Wanita Tani, Kancab Bandara SAMS Sepinggan Balikpapan Serahkan 1 Ton Pupuk Kompos - Tribunkaltim.co. https://kaltim.tribunnews.com/2022/02/17/bantu-kelompok-wanita-tani-kancab-bandara-sams-sepinggan-balikpapan-serahkan-1-ton-pupuk-kompos

Caputo, S. (2022). History, Techniques and Technologies of Soil-Less Cultivation. 45–86. https://doi.org/10.1007/978-3-030-99962-9_4

Debangshi, U. (2021). Hydroponics – An Overview. Chronicle of Bioresource Management, 5(Sep, 3), 110–114.

Gorjian, S., Calise, F., Kant, K., Ahamed, M. S., Copertaro, B., Najafi, G., Zhang, X., Aghaei, M., & Shamshiri, R. R. (2021). A review on opportunities for implementation of solar energy technologies in agricultural greenhouses. Journal of Cleaner Production, 285, 124807. https://doi.org/10.1016/J.JCLEPRO.2020.124807

Maestre-Valero, J. F., Martin-Gorriz, B., Soto-García, M., Martinez-Mate, M. A., & Martinez-Alvarez, V. (2018). Producing lettuce in soil-based or in soilless outdoor systems. Which is more economically profitable? Agricultural Water Management, 206, 48–55. https://doi.org/10.1016/J.AGWAT.2018.04.022

Marindra, A. M. J., Filiana, F., & Yuslah, N. N. (2022). PERBAIKAN DAN INSTALASI PENERANGAN JALAN UMUM TENAGA SURYA (PJUTS) DI LINGKUNGAN JALAN GIRI MULYO RT. 25 KM. 14 BALIKPAPAN. Kumawula: Jurnal Pengabdian Kepada Masyarakat, 5(2), 345–351. https://doi.org/10.24198/KUMAWULA.V5I2.37260

Majeed, Y., Khan, M. U., Waseem, M., Zahid, U., Mahmood, F., Majeed, F., Sultan, M., & Raza, A. (2023). Renewable energy as an alternative source for energy management in agriculture. Energy Reports, 10, 344–359. https://doi.org/10.1016/J.EGYR.2023.06.032

Monisha, K., Kalai Selvi, H., Sivanandhini, P., Sona Nachammai, A., Anuradha, C. T., Rama Devi, S., Kavitha Sri, A., Neya, N. R., Vaitheeswari, M., & Hikku, G. S. (2023). Hydroponics agriculture as a modern agriculture technique. Journal of Achievements in Materials and Manufacturing Engineering, Vol. 116(nr 1), 25–35. https://doi.org/10.5604/01.3001.0016.3395

Nasution, N., Lestari, S. U., & Hasan, M. A. (2021). Penerapan Teknologi Otomatisasi dalam Pertanian Agrotech Farm System. Dinamisia : Jurnal Pengabdian Kepada Masyarakat, 5(6), 1361–1373. https://doi.org/10.31849/DINAMISIA.V5I6.7752

Ng, A. K., & Mahkeswaran, R. (2021). Emerging and Disruptive Technologies for Urban Farming: A Review and Assessment. Journal of Physics: Conference Series, 2003(1), 012008. https://doi.org/10.1088/1742-6596/2003/1/012008

Pardede, G., & Pardede, G. (2022). A SWOT Analysis of the Hydroponics Entrepreneurship as Sustainable Income in Covid-19 Pandemic Adaptation. Teknotan: Jurnal Industri Teknologi Pertanian, 16(2), 93–102. https://doi.org/10.24198/jt.vol16n2.5

Prakash, C., Singh, L. P., Gupta, A., & Lohan, S. K. (2023). Advancements in smart farming: A comprehensive review of IoT, wireless communication, sensors, and hardware for agricultural automation. Sensors and Actuators A: Physical, 362, 114605. https://doi.org/10.1016/J.SNA.2023.114605

Putri, N. A., & Matarru, A. A. (2023). PEMANFAATAN LIMBAH DAUN KETAPANG MENJADI PUPUK ORGANIK CAIR UNTUK APLIKASI TANAMAN BAYAM. JMM (Jurnal Masyarakat Mandiri), 7(4), 3690–3700. https://doi.org/10.31764/JMM.V7I4.16203

Ragaveena, S., Shirly Edward, A., & Surendran, U. (2021). Smart controlled environment agriculture methods: a holistic review. Reviews in Environmental Science and Biotechnology, 20(4), 887–913. https://doi.org/10.1007/S11157-021-09591-Z/METRICS

Rozak, M., Hidayatul Arifah, A., Fitriani, R., Umardiyah, F., Muhibuddin, A., & Wahab Hasbulloh, K. A. (2022). Pelatihan Bertani Hidroponik dengan Memanfaatkan Lahan Pekarangan untuk Meningkatkan Kreativitas dan Ekonomi. Jumat Pertanian: Jurnal Pengabdian Masyarakat, 3(2), 54–58. https://doi.org/10.32764/ABDIMASPER.V3I2.2159

Saad, M. H. M., Hamdan, N. M., & Sarker, M. R. (2021). State of the Art of Urban Smart Vertical Farming Automation System: Advanced Topologies, Issues and Recommendations. Electronics 2021, Vol. 10, Page 1422, 10(12), 1422. https://doi.org/10.3390/ELECTRONICS10121422

Serio, F., Manuel, R., Machado, A., Fussy, A., & Papenbrock, J. (2022). An Overview of Soil and Soilless Cultivation Techniques—Chances, Challenges and the Neglected Question of Sustainability. Plants 2022, Vol. 11, Page 1153, 11(9), 1153. https://doi.org/10.3390/PLANTS11091153

Setyo, G., Witjaksono, B., & Si, M. (2023). Penggunaan Teknologi Tepat Guna Sistem Hidroponik dengan Memanfaatkan Lahan Kosong Di Kelurahan Rembang. Jurnal Sosiologi Pertanian Dan Agribisnis, 5(2), 43–57. https://doi.org/10.55542/JUSPA.V5I2.647